# FINDINGS OF FACT

After review of the air emission license renewal application, staff investigation reports, and other documents in the applicant’s file in the Bureau of Air Quality, pursuant to 38 Maine Revised Statutes (M.R.S.) § 344 and § 590, the Maine Department of Environmental Protection (the Department) finds the following facts:

## REGISTRATION

### Introduction

General Contractor Vaughn D Thibodeau II, Inc. (Thibodeau) has applied to renew their Air Emission License for the operation of their portable hot mix asphalt plant and crushed stone and gravel facility located at 955 Bangor Road (RT 1A), Prospect, Maine.

The main office is located at 924 Odlin Road, Bangor, Maine.

Thibodeau has requested an after-the-fact amendment to their license in order to make the following changes:

- Increase the annual production limit for the McCarter Asphalt Batch Plant to 250,000 tons/yr of asphalt;

- Remove Boiler #1 (hot oil heater), which has been removed from the facility, and from the license. Because a smaller hot oil heater, installed to replace Boiler #1, is below licensing threshold levels, the replacement unit is not addressed in this license;

- Replace the existing Rollercone Rock Crusher with a new unit manufactured in 2025;

- Retire Generator #1 from the facility. The unit previously licensed as Generator #3 (Detroit WA527311) will now be designated Generator #1; and

- Reduce the annual fuel use limit for Generator #1 to 15,000 gal/yr of distillate fuel.

### Emission Equipment

The following equipment is addressed in this Air Emission License:

Asphalt Plant

| Equipment | Process Rate (tons/hour) | Design Capacity (MMBtu/hr) | Fuel Type | Control Device(s) | Date of Manuf. |
| --- | --- | --- | --- | --- | --- |
| McCarter Asphalt Batch Plant | 200 | 35 | Propane | Baghouse | 1980 |

Heating Equipment

| Equipment | Max. Capacity (MMBtu/hr) | Fuel Type | Date of Manuf. |
| --- | --- | --- | --- |
| Boiler #1 (Hot Oil Heater)* | 1.4 | Distillate Fuel, Spec. Waste Oil | 2000 |

*Removed from license

Rock Crushers

| Designation | Powered | Process Rate (tons/hour) | Date of Manufacture | Date of Installation | Control Device |
| --- | --- | --- | --- | --- | --- |
| Jaw #1 | Generator #1 | 100 | 2004 | 2010 | Spray Nozzles |
| Rollercone #1* | Generator #1 | 100 | 2025 | 2025 | Spray Nozzles |
| Rollercone-Crusher Serial #42DOJ95 ** | Generator #1 | 100 | 1990 | -- | Spray Nozzles |

* New to license

** Removed from license

Engines

| Unit ID | Max. Capacity (MMBtu/hr) | Max. Firing Rate (gal/hr) | Fuel Type | Date of Manuf. |
| --- | --- | --- | --- | --- |
| Generator #1* (Serial # WA527311) | 4.96 | 35.7 | Distillate Fuel | 2000 |
| Generator #1** | 3.2 | 23.4 | Distillate Fuel | 1999 |

*Previously licensed as ‘Generator #3’

**Removed from license

Thibodeau may operate other nonmetallic mineral processing equipment not explicitly listed including grinding mills, screening operations, bucket elevators, belt conveyors, bagging operations, storage bins, and enclosed truck or railcar loading stations. Requirements for this equipment are included in sections of this license for Nonmetallic Mineral Processing Plants.

Thibodeau may operate small stationary engines smaller than 0.5 MMBtu/hr. These engines are considered insignificant activities and are not required to be included in this license. However, they are still subject to applicable State and Federal regulations. More information regarding requirements for small stationary engines is available on the Department’s website at the link below.

http://www.maine.gov/dep/air/publications/docs/SmallRICEGuidance.pdf

Additionally, Thibodeau may operate portable engines used for maintenance or emergency-only purposes. These engines are considered insignificant activities and are not required to be included in this license. However, they may still be subject to applicable State and Federal regulations.

### Definitions

Distillate Fuel means the following:

- Fuel oil that complies with the specifications for fuel oil numbers 1 or 2, as defined by the American Society for Testing and Materials (ASTM) in ASTM D396;

- Diesel fuel oil numbers 1 or 2, as defined in ASTM D975;

- Kerosene, as defined in ASTM D3699;

- Biodiesel, as defined in ASTM D6751; or

- Biodiesel blends, as defined in ASTM D7467.

Nonmetallic mineral processing plant means any combination of equipment that is used to crush or grind any nonmetallic mineral wherever located, including lime plants, power plants, steel mills, asphalt concrete plants, portland cement plants (not including concrete batch plants), or any other facility processing nonmetallic minerals.

Portable or Non-Road Engine means an internal combustion engine which is portable or transportable, meaning designed to be and capable of being carried or moved from one location to another. Indicia of transportability include, but are not limited to, wheels, skids, carrying handles, dolly, trailer, or platform. This definition does NOT include engines which remain or will remain at a location (excluding storage locations) for more than 12 consecutive months or a shorter period of time for an engine located at a seasonal source. A location is any single site at a building, structure, facility, or installation. Any engine that replaces an engine at a location and that is intended to perform the same or similar function as the engine replaced will be included in calculating the consecutive time period.

An engine is not a non-road (portable) engine if it remains or will remain at a location for more than 12 consecutive months or for a shorter period of time if sited at a seasonal source. A seasonal source is a source that remains in a single location for two years or more and which operates for fewer than 12 months in a calendar year. If an engine operates at a seasonal source for one entire season, the engine does not meet the criteria of a non-road (portable) engine and is subject to applicable stationary engine requirements.

Records or Logs mean either hardcopy or electronic records.

### Application Classification

All rules, regulations, or statutes referenced in this air emission license refer to the amended version in effect as of the date this license was issued.

Thibodeau has applied to renew currently licensed emission units as well as modify their license as addressed in Section I(A) above.

The modification of a minor source is considered a major or minor modification based on whether or not expected emission increases exceed the “Significant Emissions” levels as defined in the Department’s Definitions Regulation, 06-096 Code of Maine Rules (C.M.R.) ch. 100. The emission increases are determined by subtracting the current licensed annual emissions preceding the modification from the maximum future licensed annual emissions, as follows:

| Pollutant | Current License (tpy) | Future License (tpy) | Net Change (tpy) | Significant Emission Levels |
| --- | --- | --- | --- | --- |
| PM | 8.1 | 7.5 | -0.6 | 100 |
| PM10 | 8.1 | 7.5 | -0.6 | 100 |
| PM2.5 | 8.1 | 7.5 | -0.6 | 100 |
| SO2 | 10.9 | 0.6 | -10.3 | 100 |
| NOx | 24.4 | 6.4 | -18.0 | 100 |
| CO | 37.7 | 50.9 | +13.2 | 100 |
| VOC | 1.9 | 1.1 | -0.8 | 100 |

Therefore, this license is considered to be both a renewal and a minor modification and has been processed through Major and Minor Source Air Emission License Regulations, 06-096 Code of Maine Rules C.M.R. ch. 115.

### Facility Classification

With the annual fuel use limit of 15,000 gallons of distillate fuel on Generator #1, and the 250,000-ton annual production limit for the McCarter Asphalt Batch Plant, the facility is licensed as follows:

- As a synthetic minor source of air emissions for criteria pollutants, because Thibodeau is subject to license restrictions that keep facility emissions below major source thresholds for carbon monoxide (CO); and

- As an area source of hazardous air pollutants (HAP), because the licensed emissions are below the major source thresholds for HAP.

## BEST PRACTICAL TREATMENT

### Introduction

In order to receive a license, the applicant must control emissions from each unit to a level considered by the Department to represent Best Practical Treatment (BPT), as defined in Definitions Regulation, 06-096 C.M.R. ch. 100. Separate control requirement categories exist for new and existing equipment.

BPT for new sources and modifications requires a demonstration that emissions are receiving Best Available Control Technology (BACT), as defined in 06-096 C.M.R. ch. 100. BACT is a top-down approach to selecting air emission controls considering economic, environmental, and energy impacts.

BPT for existing emissions equipment means that method which controls or reduces emissions to the lowest possible level considering:

- the existing state of technology;

- the effectiveness of available alternatives for reducing emissions from the source being considered; and

- the economic feasibility for the type of establishment involved.

### McCarter Asphalt Batch Plant

Thibodeau operates a the portable McCarter Asphalt Batch Plant with a maximum hourly throughput of 200 ton/hr of asphalt and a 35 MMBtu/hr burner which fires propane.

Emission factors for asphalt plants are available based on tons of asphalt produced, and there is no linear relationship between plant output and burner firing rate. Therefore, to ensure annual emissions are limited to less than major source thresholds, asphalt throughput is limited instead of fuel consumption. Accordingly, the annual throughput of the asphalt plant shall not exceed 250,000 tons of asphalt per year on a 12-month rolling total basis.

##### BPT Findings

Thibodeau operates the McCarter Asphalt Batch Plant with a baghouse. A baghouse is a pollution control device that captures and removes PM/PM10/PM2.5 emissions from the gas stream by depositing the PM/PM10/PM2.5 emissions on fabric material bags. The collected particulate matter is shaken from the bags and removed for disposal.

The BPT emission limits for the McCarter Asphalt Batch Plant were based on the following:

| PM/PM10/PM2.5 | – | 0.03 gr/dscf and the use of a baghouse pursuant to 06-096 C.M.R. ch. 115, BACT |
| --- | --- | --- |
| SO2 | – | 4.6 x 10-3 lb/ton based on AP-42 Table 11.1-5 dated 3/04 |
| NOx | – | 2.5 x 10-2 lb/ton based on AP-42 Table 11.1-5 dated 3/04 |
| CO | – | 0.40 lb/ton based on AP-42 Table 11.1-5 dated 3/04 |
| VOC | – | 8.2 x 10-3 lb/ton based on AP-42 Table 11.1-6 dated 3/04 |
| Visible Emissions | – | 06-096 C.M.R. ch. 101 |

The BPT emission limits for the McCarter Asphalt Batch Plant are the following:

| ### Unit | PM (lb/hr) | PM10 (lb/hr) | PM2.5 (lb/hr) | SO2 (lb/hr) | NOx (lb/hr) | CO (lb/hr) | VOC (lb/hr) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| McCarter Asphalt Batch Plant | 11.75 | 11.75 | 11.75 | 0.92 | 5.00 | 80.00 | 1.64 |

Visible emissions from McCarter Asphalt Batch Plant baghouse shall not exceed 20% opacity on a six-minute block average basis. This is consistent with the PM limit of 20% opacity contained in Standards of Performance for Hot Mix Asphalt Facilities, 40 C.F.R. Part 60, Subpart I.

##### New Source Performance Standards

The McCarter Asphalt Batch Plant was manufactured in 1980 and is therefore subject to the federal Environmental Protection Agency’s (EPA) New Source Performance Standards (NSPS) Standards of Performance for Hot Mix Asphalt Facilities, 40 Code of Federal Regulation (C.F.R.) Part 60, Subpart I for facilities constructed or modified after June 11, 1973.

- Notification

Thibodeau was required to submit notification to EPA and the Department of the date of initial startup. [40 C.F.R. § 60.7(a)(3)]

- Standards

- Particulate Matter (PM)

The McCarter Asphalt Batch Plant shall not exceed an emission limit of 0.04 gr/dscf. [40 C.F.R. § 60.92(a)(1)]

The Department has determined that the proposed BACT PM emission limit is more stringent than the applicable limit in 40 C.F.R. Part 60, Subpart I. Therefore, the PM limit for the asphalt plant has been streamlined to the more stringent BACT limit, and only this more stringent limit shall be included in the Order of this air emission license.

- Opacity

Visible emissions from the asphalt plant shall not exceed 20% opacity on a six-minute block average basis. [40 C.F.R. §§ 60.92(a)(2) and 60.93(b)(2)] This standard applies at all times. [06-096 C.M.R. ch.101, § 4(B)(1)]

- Initial Compliance Requirements

Thibodeau is required to perform the following within 60 days after achieving the maximum production rate at which the asphalt plant will be operated but not later than 180 days after the initial startup:

- Thibodeau shall conduct an initial performance test for PM using 40 C.F.R. Part 60, Appendix A, Method 5. [40 C.F.R. § 60.93(b)(1)]

- Thibodeau shall conduct an initial performance test for opacity using 40 C.F.R. Part 60, Appendix A, Method 9. [40 C.F.R. § 60.93(b)(2)]

Because the unit was installed prior to being added to the air emission license, Thibodeau must comply with one of the following:

- Provide the Department documentation of the date(s) these tests were conducted and demonstrated that standards are being met; or

- Conduct these tests as soon as possible and provide documentation to the Department of the successfully completed tests.

[06-096 C.M.R. ch. 115, BPT]

For any performance testing required by this license, Thibodeau shall submit to the Department for approval a performance test protocol, as outlined in the Department’s Performance Testing Guidance, at least 30 days prior to the scheduled date of the performance test. [06-096 C.M.R. ch. 115, BPT]

Standard Condition (8) requires the facility to maintain records for a minimum of six (6) years. The McCarter Asphalt Batch Plant is an existing piece of equipment installed over six years ago; thus records of initial notification and initial compliance demonstration are outside of the required record retention period.

Note: Although some federal standards, such as 40 C.F.R. Part 60, Subpart OOO, allow for a shorter pretest notification period, the Department requires pretest notification a minimum of 30 days prior to the scheduled date of the performance test unless a variance of this requirement is preapproved by the Department.

The Department’s Performance Testing Guidance is available online at: https://www.maine.gov/dep/air/emissions/testing.html

##### Control Equipment

Emissions from the McCarter Asphalt Batch Plant shall be controlled by a baghouse.

##### Periodic Monitoring

The performance of the baghouse shall be monitored by either one of the following at all times the asphalt plant is operating:

- Continuous PM Detector:

When the detector signals excessive PM concentrations in the exhaust stream, Thibodeau shall take corrective action within 24 hours, or immediately if visible emissions exceed 20% opacity.

- Personnel

Personnel shall be available on-site with a current EPA 40 C.F.R. Part 60, Appendix A, Method 9 visible emissions certification: When visible emissions exceed 20% opacity, the hot mix asphalt plant is operating with insufficient control, and corrective action shall be taken immediately.

Thibodeau shall keep records of baghouse failures, baghouse maintenance, and baghouse inspections.

To document maintenance of the baghouse, Thibodeau shall keep records of the date and location of all bag failures, the date and a description of all routine and non-routine maintenance, and the date and results of all inspections. These records shall be kept on-site at the asphalt plant location.

##### Contaminated Soils

- Soils Contaminated with Gasoline and Distillate Fuel

Thibodeau may process up to 10,000 cubic yards per calendar year of soil contaminated by gasoline or distillate fuel without prior approval from the Department’s Bureau of Air Quality.

This limit may be exceeded with prior written authorization from the Department’s Bureau of Air Quality. Requests will be evaluated on a case-by-case basis taking into account the nature and amount of the contaminated soil to be processed, the location where the processing will occur, and the potential for fugitive emissions.

- General Requirements for Processing of Contaminated Soils

Thibodeau shall not process soils which are classified as hazardous waste or which have unknown contaminants.

Thibodeau shall notify the Department (regional air compliance inspector) at least 24 hours prior to processing the contaminated soil and specify the contaminating material and quantity, origin of the soil and contaminating material, and the disposition of the contaminated soil. This authorization to process contaminated soil does not absolve the facility of responsibility to comply with all other air emission license conditions and any other applicable state rules or statutes.

When processing contaminated soils, Thibodeau shall maintain records which specify the quantity and type of contaminant in the soil as well as the origin and characterization of the contaminated soil. In addition, when processing contaminated soil, Thibodeau shall maintain records on an hourly basis of processing temperature, asphalt feed rates, and dryer throughput.

Any approval from the Department’s Bureau of Air Quality to process contaminated soil does not supersede requirements from other Department bureaus. Similarly, approvals to process contaminated soil granted by another Bureau of the  Department does not supersede the limits imposed by this air emission license.

### Rock Crushers Jaw #1 and Rollercone #1

Rock Crushers Jaw #1 and Rollercone #1 are portable units which were manufactured in 2004 and 2025, respectively, with rated capacity of 100 tons/hr each. The nonmetallic mineral processing plant also consists of other equipment associated with Rock Crushers Jaw #1 and Rollercone #1, such as screens and belt conveyors.

##### BACT (Rollercone #1 Crusher) and BPT (Jaw #1 Crusher) Findings

The regulated pollutant from nonmetallic mineral processing plants is particulate matter. To meet the requirements of BACT for control of particulate matter emissions, Thibodeau shall install and maintain water sprays on the nonmetallic mineral processing plants and operate as needed, when the units are in operation, to control visible emissions.

##### Visible Emissions

Visible emissions from Rock Crushers Jaw #1 and Rollercone #1 shall each be limited to no greater than 10% opacity on a six-minute block average basis. [06-096 C.M.R. ch. 101, § 4(B)(2)]

Visible emissions from nonmetallic mineral processing plant equipment other than crushers (transfer points on belt conveyors, screening operations, etc.) shall not exceed 20% opacity on a six-minute block average basis. [06-096 C.M.R. ch. 101, § 4(B)(4)]

##### New Source Performance Standards

The federal regulation Standards of Performance for Nonmetallic Mineral Processing Plants, 40 C.F.R. Part 60, Subpart OOO, applies to equipment at nonmetallic mineral processing plants with capacities greater than 25 ton/hr for fixed plants and 150 ton/hr for portable plants. The requirements of Subpart OOO apply to any crusher, grinding mill, screening operation, bucket elevator, belt conveyor, bagging operation, storage bin, or enclosed truck or railcar loading station at a nonmetallic mineral processing plant greater than the sizes listed above which commenced construction, modification, or reconstruction after August 31, 1983.

Rock Crushers Jaw #1 and Rollercone #1 are portable with rated capacities of 100 tons/hr each. Therefore, this equipment is not subject to 40 C.F.R. Part 60, Subpart OOO. [40 C.F.R. § 60.670(c)]

### Generator #1

Generator #1 is a portable engine used as back-up power for the asphalt batch plant equipment. Generator #1 has a maximum capacity of 4.96 MMBtu/hr, firing distillate fuel. The generator was manufactured in 2000 and is equipped with a Detroit Engine Model #WA527311. The fuel fired in Generator #1 shall be limited to 15,000 gallons/year on a calendar year total basis of distillate fuel with a maximum sulfur content not to exceed 15 ppm (0.0015% sulfur by weight). This fuel limit shall apply regardless of where the unit is operated.

##### BPT Findings

The BPT emission limits for Generator #1 were based on the following:

| PM/PM10/PM2.5 | – | 0.12 lb/MMBtu from 06-096 C.M.R. ch. 103 |
| --- | --- | --- |
| SO2 | – | Combustion of distillate fuel with a maximum sulfur content not to exceed 15 ppm (0.0015% sulfur by weight) |
| NOx | – | 3.2 lb/MMBtu from AP-42 Table 3.4-1 dated 4/25 |
| CO | – | 0.85 lb/MMBtu from AP-42 Table 3.4-1 dated 4/25 |
| VOC | – | 0.09 lb/MMBtu from AP-42 Table 3.4-1 dated 4/25 |
| Visible Emissions | – | 06-096 C.M.R. ch. 101 |

The BPT emission limits for Generator #1 are the following:

| Unit | Pollutant | lb/MMBtu |
| --- | --- | --- |
| Generator #1 | PM | 0.12 |

| ### Unit | PM (lb/hr) | PM10 (lb/hr) | PM2.5 (lb/hr) | SO2 (lb/hr) | NOx (lb/hr) | CO (lb/hr) | VOC (lb/hr) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Generator #1 | 0.60 | 0.60 | 0.60 | 0.01 | 15.87 | 4.22 | 0.45 |

Visible emissions from Generator #1 shall not exceed 20% opacity on a six-minute block average basis except for periods of startup during which time Thibodeau shall either meet the normal operating visible emissions standard or the following work practice standards and alternative visible emissions standard.

- The duration of the startup shall not exceed 30 minutes per event;

- Visible emissions shall not exceed 50% opacity on a six-minute block average basis; and

- Thibodeau shall keep records of the date, time, and duration of each startup.

Use of the work practice standards and alternative visible emissions standard in lieu of the normal operating standard is limited to no more than once per day.

Note: This does not limit the engine to one startup per day. It only limits the use of the alternative emission standard to once per day.

##### New Source Performance Standards

Generator #1 is not subject to Standards of Performance for Stationary Compression Ignition Internal Combustion Engines, 40 C.F.R. Part 60, Subpart IIII.

The definition in 40 C.F.R. § 1068.30 states that a non-road engine is an internal combustion engine that meets certain criteria, including: “Portable or transportable, meaning designed to be and capable of being carried or moved from one location to another. Indicia of transportability include, but are not limited to, wheels, skids, carrying handles, dolly, trailer, or platform.” The regulation further states at 40 C.F.R. § 1068.30 that an engine is not a non-road engine if it remains or will remain at a location for more than 12 consecutive months or for a shorter period of time if sited at a seasonal source. A seasonal source is a source that remains in a single location for two years or more and which operates for fewer than 12 months in a calendar year. If an engine operates at a seasonal source for one entire season, the engine does not meet the criteria of a non-road engine and is subject to applicable stationary engine requirements.  [40 C.F.R. § 60.4200]

Generator #1 is considered a non-road engine, as opposed to a stationary engine, since Generator #1 is portable and will be moved to various sites with the asphalt plant.

##### National Emission Standards for Hazardous Air Pollutants

Generator #1 is not subject to National Emission Standards for Hazardous Air Pollutants for Stationary Reciprocating Internal Combustion Engines, 40 C.F.R. Part 63, Subpart ZZZZ.

The definition in 40 C.F.R. § 1068.30 states that a non-road engine is an internal combustion engine that meets certain criteria, including: “Portable or transportable, meaning designed to be and capable of being carried or moved from one location to another. Indicia of transportability include, but are not limited to, wheels, skids, carrying handles, dolly, trailer, or platform.” The regulation further states at 40 C.F.R. § 1068.30 that an engine is not a non-road engine if it remains or will remain at a location for more than 12 consecutive months or for a shorter period of time if sited at a seasonal source. A seasonal source is a source that remains in a single location for two years or more and which operates for fewer than 12 months in a calendar year. If an engine operates at a seasonal source for one entire season, the engine does not meet the criteria of a non-road engine and is subject to applicable stationary engine requirements. [40 C.F.R. § 63.6585]

Generator #1 is considered a non-road engine, as opposed to a stationary engine, since Generator #1 is portable and will be moved to various sites with the asphalt plant.

### General Process Emissions

Visible emissions from any general process that is not part of a nonmetallic mineral processing plant shall not exceed 20% opacity on a six-minute block average basis.

### Fugitive Emissions Including Stockpiles and Roadways

Thibodeau shall not cause emissions of any fugitive dust during any period of construction, reconstruction, or operation without taking reasonable precautions. Such reasonable precautions shall be included in the facility’s continuing program of best management practices for suppression of fugitive particulate matter. See 06-096 C.M.R. ch. 101, § 4(C) for a list of potential reasonable precautions.

Thibodeau shall not cause or allow visible emissions within 20 feet of ground level, measured as any level of opacity and not including water vapor, beyond the legal boundary of the property on which such emissions occur. Compliance with this standard shall be determined pursuant to 40 C.F.R. Part 60, Appendix A, Method 22.

### Annual Emissions

The table below provides an estimate of facility-wide annual emissions for the purposes of calculating the facility’s annual air license fee and establishing the facility’s potential to emit (PTE). Only licensed equipment is included, i.e., emissions from insignificant activities are excluded. Similarly, unquantifiable fugitive particulate matter emissions are not included except when required by state or federal regulations. Maximum potential emissions were calculated based on the following assumptions:

- Processing 250,000 ton/year of asphalt; and

- Firing 15,000 gal/year of distillate fuel in Generator #1.

This information does not represent a comprehensive list of license restrictions or permissions. That information is provided in the Order section of this license.

Total Licensed Annual Emissions for the Facility

Tons/year

(used to calculate the annual license fee)

|  | PM | PM10 | PM2.5 | SO2 | NOx | CO | VOC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| McCarter Asphalt Batch Plant | 7.3 | 7.3 | 7.3 | 0.6 | 3.1 | 50.0 | 1.0 |
| Generator #1 | 0.2 | 0.2 | 0.2 | -- | 3.3 | 0.9 | 0.1 |
| Total TPY | 7.5 | 7.5 | 7.5 | 0.6 | 6.4 | 50.9 | 1.1 |

| Pollutant | Tons/year |
| --- | --- |
| Single HAP | 7.9 |
| Total HAP | 19.9 |

## AMBIENT AIR QUALITY ANALYSIS

The level of ambient air quality impact modeling required for a minor source to demonstrate that Ambient Air Quality Standards (AAQS) will not be exceeded is determined by the Department on a case-by-case basis. In accordance with 06-096 C.M.R. ch. 115, an ambient air quality impact analysis is not required for a minor source if the total licensed annual emissions of any pollutant released do not exceed the following levels and there are no extenuating circumstances:

| Pollutant | Tons/Year |
| --- | --- |
| PM10 | 25 |
| PM2.5 | 15 |
| SO2 | 50 |
| NOx | 50 |
| CO | 250 |

The total licensed annual emissions for the facility are below the emission levels contained in the table above and there are no extenuating circumstances; therefore, an ambient air quality impact analysis is not required as part of this license renewal and minor revision.

This determination is based on information provided by the applicant regarding the expected construction and operation of the proposed and currently licensed emission units. If the Department determines that any parameter (e.g., stack size, configuration, flow rate, emission rates, nearby structures, etc.) deviates from what was included in the application, the Department may require Thibodeau to submit additional information and may require an ambient air quality impact analysis at that time.

# ORDER

Based on the above Findings and subject to conditions listed below, the Department concludes that the emissions from this source:

- will receive Best Practical Treatment,

- will not violate applicable emission standards,

- will not violate applicable ambient air quality standards in conjunction with emissions from other sources.

The Department hereby grants Air Emission License Renewal and Minor Revision A-1045-71-F-R/A, subject to the following conditions.

Severability. The invalidity or unenforceability of any provision of this License Renewal or part thereof shall not affect the remainder of the provision or any other provisions. This License Renewal shall be construed and enforced in all respects as if such invalid or unenforceable provision or part thereof had been omitted.

## STANDARD CONDITIONS

#### Employees and authorized representatives of the Department shall be allowed access to the licensee’s premises during business hours, or any time during which any emissions units are in operation, and at such other times as the Department deems necessary for the purpose of performing tests, collecting samples, conducting inspections, or examining and copying records relating to emissions (38 M.R.S. § 347-C).

#### The licensee shall acquire a new or amended air emission license prior to beginning actual construction of a modification, unless specifically provided for in 06-096 C.M.R. ch. 115. [06-096 C.M.R. ch. 115]

#### Approval to construct shall become invalid if the source has not commenced construction within eighteen (18) months after receipt of such approval or if construction is discontinued for a period of eighteen (18) months or more. The Department may extend this time period upon a satisfactory showing that an extension is justified, but may condition such extension upon a review of either the control technology analysis or the ambient air quality standards analysis, or both. [06-096 C.M.R. ch. 115]

#### The licensee shall establish and maintain a continuing program of best management practices for suppression of fugitive particulate matter during any period of construction, reconstruction, or operation which may result in fugitive dust, and shall submit a description of the program to the Department upon request. [06-096 C.M.R. ch. 115]

#### The licensee shall pay the annual air emission license fee to the Department, calculated pursuant to Title 38 M.R.S. § 353-A. [06-096 C.M.R. ch. 115] Payment of the annual air emission license fee for Thibodeau is due by the end of February of each year. [38 M.R.S. § 353-A(3)]

#### The license does not convey any property rights of any sort or any exclusive privilege. [06-096 C.M.R. ch. 115]

#### The licensee shall maintain and operate all emission units and air pollution systems required by the air emission license in a manner consistent with good air pollution control practice for minimizing emissions. [06-096 C.M.R. ch. 115]

#### The licensee shall maintain sufficient records to accurately document compliance with emission standards and license conditions and shall maintain such records for a minimum of six (6) years. The records shall be submitted to the Department upon written request. [06-096 C.M.R. ch. 115]

#### The licensee shall comply with all terms and conditions of the air emission license. The filing of an appeal by the licensee, the notification of planned changes or anticipated noncompliance by the licensee, or the filing of an application by the licensee for a renewal of a license or amendment shall not stay any condition of the license. [06-096 C.M.R. ch. 115]

#### The licensee may not use as a defense in an enforcement action that the disruption, cessation, or reduction of licensed operations would have been necessary in order to maintain compliance with the conditions of the air emission license. [06-096 C.M.R. ch. 115]

#### In accordance with the Department’s air emission compliance test protocol and 40 C.F.R. Part 60 or other method approved or required by the Department, the licensee shall:

- Perform stack testing to demonstrate compliance with the applicable emission standards under circumstances representative of the facility’s normal process and operating conditions:

- Within sixty (60) calendar days of receipt of a notification to test from the Department or EPA, if visible emissions, equipment operating parameters, staff inspection, air monitoring, or other cause indicate to the Department that equipment may be operating out of compliance with emission standards or license conditions; or

- Pursuant to any other requirement of this license to perform stack testing.

- Install or make provisions to install test ports that meet the criteria of 40 C.F.R. Part 60, Appendix A, and test platforms, if necessary, and other accommodations necessary to allow emission testing; and

- Submit a written report to the Department within thirty (30) days from date of test completion.

[06-096 C.M.R. ch. 115]

#### If the results of a stack test performed under circumstances representative of the facility’s normal process and operating conditions indicate emissions in excess of the applicable standards, then:

- Within thirty (30) days following receipt of the written test report by the Department, or another alternative timeframe approved by the Department, the licensee shall re-test the non-complying emission source under circumstances representative of the facility’s normal process and operating conditions and in accordance with the Department’s air emission compliance test protocol and 40 C.F.R. Part 60 or other method approved or required by the Department; and

- The days of violation shall be presumed to include the date of stack test and each and every day of operation thereafter until compliance is demonstrated under normal and representative process and operating conditions, except to the extent that the facility can prove to the satisfaction of the Department that there were intervening days during which no violation occurred or that the violation was not continuing in nature; and

- The licensee may, upon the approval of the Department following the successful demonstration of compliance at alternative load conditions, operate under such alternative load conditions on an interim basis prior to a demonstration of compliance under normal and representative process and operating conditions.

[06-096 C.M.R. ch. 115]

#### Notwithstanding any other provisions in the State Implementation Plan approved by the EPA or Section 114(a) of the CAA, any credible evidence may be used for the purpose of establishing whether a person has violated or is in violation of any statute, regulation, or license requirement. [06-096 C.M.R. ch. 115]

#### The licensee shall maintain records of malfunctions, failures, downtime, and any other similar change in operation of air pollution control systems or the emissions unit itself that would affect emissions and that is not consistent with the terms and conditions of the air emission license. The licensee shall notify the Department within two (2) days or the next state working day, whichever is later, of such occasions where such changes result in an increase of emissions. The licensee shall report all excess emissions in the units of the applicable emission limitation. [06-096 C.M.R. ch. 115]

#### Upon written request from the Department, the licensee shall establish and maintain such records; make such reports; install, use, and maintain such monitoring equipment; sample such emissions in accordance with such methods, at such locations, at such intervals, and in such a manner as the Department shall prescribe; and provide other information as the Department may reasonably require to determine the licensee’s compliance status. 
[06-096 C.M.R. ch. 115]

#### The licensee shall notify the Department within 48 hours and submit a report to the Department on a quarterly basis if a malfunction or breakdown in any component causes a violation of any emission standard (38 M.R.S. § 605). [06-096 C.M.R. ch. 115]

## SPECIFIC CONDITIONS

#### McCarter Asphalt Batch Plant

- The McCarter Asphalt Batch Plant is licensed to fire propane. [06-096 C.M.R. ch. 115, BPT]

- Thibodeau shall maintain records which demonstrate that the McCarter Asphalt Batch Plant is relocated and operated on a basis which maintains its classification of portable.

[06-096 C.M.R. ch. 115, BPT]

- The annual throughput of the McCarter Batch Asphalt Plant shall not exceed 250,000 tons of asphalt per year on a 12-month rolling total basis. Records of asphalt productions shall be kept on a monthly and 12-month rolling total basis. [06-096 C.M.R. ch. 115, BPT]

- Emissions from the asphalt plant shall vent to a baghouse, and all components of the asphalt plant shall be maintained so as to prevent PM leaks. [06-096 C.M.R. ch. 115, BPT]

- The performance of the baghouse shall be monitored by either one of the following at all times the McCarter Batch Asphalt Plant is operating:

[06-096 C.M.R. ch. 115, BPT]

- Continuous PM Detector:

When the detector signals excessive PM concentrations in the exhaust stream, Thibodeau shall take corrective action within 24 hours, or immediately if opacity exceeds 20%.

- Personnel

Personnel shall be available on-site with a current EPA Method 9 visible emissions certification: When visible emissions exceed 20% opacity, the asphalt plant is operating with insufficient control, and corrective action shall be taken immediately.

- To document maintenance of the baghouse, Thibodeau shall keep records of the date and location of all bag failures, the date and a description of all routine and non-routine maintenance, and the date and results of all inspections. These records shall be kept on-site at the asphalt plant location. [06-096 C.M.R. ch. 115, BPT]

- Emissions from the asphalt plant baghouse shall not exceed the following [06-096 C.M.R. ch. 115, BPT]:

| Pollutant | grs/dscf | lb/hr |
| --- | --- | --- |
| PM | 0.03 | 11.75 |
| PM10 | – | 11.75 |
| PM2.5 | – | 11.75 |
| SO2 | – | 0.92 |
| NOx | – | 5.00 |
| CO | – | 80.00 |
| VOC | – | 1.64 |

- General process emissions from the hot mix asphalt plant shall be controlled so as to prevent visible emissions in excess of 20% opacity on a six-minute block average basis. [06-096 C.M.R. ch. 101, § 4(B)(4)]

- Thibodeau shall comply with all requirements of 40 C.F.R. Part 60, Subpart I applicable to the asphalt plant including, but not limited to, the following:

- Visible emissions from the asphalt plant shall not exceed 20% opacity on a 6-minute block average basis. [40 C.F.R. §§ 60.92(a)(2) and 60.93(b)(2)] This standard applies at all times. [06-096 C.M.R. ch. 101, § 4(B)(1)]

- Initial Compliance Requirements of 40 C.F.R. Part 60, Subpart I

Thibodeau was required to perform the following within 60 days after achieving the maximum production rate at which the asphalt plant will be operated but not later than 180 days after the initial startup:

- Thibodeau shall conduct an initial performance test for PM using 40 C.F.R. Part 60, Appendix A, Method 5. [40 C.F.R. § 60.93(b)(1)]

- Thibodeau shall conduct an initial performance test for opacity using 40 C.F.R. Part 60, Appendix A, Method 9. [40 C.F.R. § 60.93(b)(2)]

Because this unit was installed prior to being added to the air emission license, Thibodeau must promptly comply with one of the following:

- Provide to the Department documentation of the date(s) these tests were conducted and demonstrated that standards are being met; or

- Conduct these tests as soon as possible and provide documentation to the Department of the successfully completed tests.

[06-096 C.M.R. ch. 115, BPT]

For any performance testing required by this license, Thibodeau shall submit to the Department for approval a performance test protocol, as outlined in the Department’s Performance Testing Guidance, at least 30 days prior to the scheduled date of the performance test. [06-096 C.M.R. ch. 115, BPT]

- Contaminated Soils

- Soils Contaminated with Gasoline and Distillate Fuel

Thibodeau may process up to 10,000 cubic yards per calendar year of soil contaminated by gasoline or distillate fuel without prior approval from the Department’s Bureau of Air Quality.

This limit may be exceeded with prior written authorization from the Department’s Bureau of Air Quality. Requests will be evaluated on a case-by-case basis taking into account the nature and amount of the contaminated soil to be processed, the location where the processing will occur, and the potential for fugitive emissions.

- General Requirements for Contaminated Soils

- Thibodeau shall not process soils which are classified as hazardous waste or which have unknown contaminants.

- Thibodeau shall notify the Department (regional air compliance inspector) at least 24 hours prior to processing the contaminated soil and specify the contaminating material and quantity, origin of the soil and contaminating material, and the disposition of the contaminated soil. This authorization to process contaminated soil does not absolve the facility of responsibility to comply with all other air emission license conditions and any other applicable state rules or statutes.

- When processing contaminated soils, Thibodeau shall maintain records which specify the quantity and type of contaminant in the soil as well as the origin and characterization of the contaminated soil. In addition, when processing contaminated soil, Thibodeau shall maintain records on an hourly basis of processing temperature, asphalt feed rates, and dryer throughput.

[06-096 C.M.R. ch. 115, BPT]

#### Rock Crushers Jaw #1 and Rollercone #1

- Thibodeau shall install and maintain spray nozzles for control of particulate matter on the nonmetallic mineral processing plants and operate as needed, when the units are in operation, to control visible emissions. [06-096 C.M.R. ch. 115, BPT and BACT]

- Thibodeau shall maintain records of the dates and times of all operating hours for Jaw #1 and Rollercone #1. The operation records shall be kept on-site at the rock crushing location. [06-096 C.M.R. ch. 115, BPT and BACT]

- Visible emissions from Rock Crushers Jaw #1 and Rollercone #1 shall each be limited to no greater than 10% opacity on a six-minute block average basis. [06-096 C.M.R. ch. 101, § 4(B)(2)]

- Visible emissions from nonmetallic mineral processing plant equipment other than crushers (transfer points on belt conveyors, screening operations, etc.) shall not exceed 20% opacity on a six-minute block average basis. [06-096 C.M.R. ch. 101, § 4(B)(4)]

#### Generator #1

- Fuel Use

- Generator #1 is licensed to fire distillate fuel with a maximum sulfur content not to exceed 15 ppm (0.0015% sulfur by weight). Compliance shall be demonstrated by fuel delivery receipts from the supplier, fuel supplier certification, certificate of analysis, or testing of fuel in the tank on-site. [06-096 C.M.R. ch. 115, BPT]

- Total fuel use for Generator #1 shall not exceed 15,000 gal/yr of distillate fuel, regardless of where the unit is operated. Compliance shall be demonstrated by fuel records from the supplier showing the quantity and type of fuel delivered. Records of annual fuel use shall be kept on a monthly and calendar year basis. [06-096 C.M.R. ch. 115, BPT]

- Thibodeau shall maintain records which demonstrate that Generator #1 is relocated and operated on a basis which maintains the classification of a non-road portable engine. [06-096 C.M.R. ch. 115, BPT]

- Emissions shall not exceed the following:

| Unit | Pollutant | lb/MMBtu | Origin and Authority |
| --- | --- | --- | --- |
| Generator #1 | PM | 0.12 | 06-096 C.M.R. ch. 103, § (2)(B)(1)(a) |

- Emissions shall not exceed the following [06-096 C.M.R. ch. 115, BPT]:

| ### Unit | PM (lb/hr) | PM10 (lb/hr) | PM2.5 (lb/hr) | SO2 (lb/hr) | NOx (lb/hr) | CO (lb/hr) | VOC (lb/hr) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Generator #1 | 0.60 | 0.60 | 0.60 | 0.01 | 15.87 | 4.22 | 0.45 |

- Visible Emissions

Visible emissions from Generator #1 shall not exceed 20% opacity on a six-minute block average basis except for periods of startup during which time Thibodeau shall either meet the normal operating visible emissions standard or the following work practice standards and alternative visible emissions standard.

- The duration of the startup shall not exceed 30 minutes per event;

- Visible emissions shall not exceed 50% opacity on a six-minute block average basis; and

- Thibodeau shall keep records of the date, time, and duration of each startup.

Use of the work practice standards and alternative visible emissions standard in lieu of the normal operating standard is limited to no more than once per day.

Note: This does not limit the engine to one startup per day. It only limits the use of the alternative emission standard to once per day.

[06-096 C.M.R. ch. 101, § 4(A)(4)]

#### General Process Sources

Visible emissions from any general process that is not part of a nonmetallic mineral processing plant shall not exceed 20% opacity on a six-minute block average basis.

[06-096 C.M.R. ch. 101, § 4(B)(4)]

#### Fugitive Emissions Including Stockpiles and Roadways

Thibodeau shall not cause emissions of any fugitive dust during any period of construction, reconstruction, or operation without taking reasonable precautions. Such reasonable precautions shall be included in the facility’s continuing program of best management practices for suppression of fugitive particulate matter. See 06-096 C.M.R. ch. 101, § 4(C) for a list of potential reasonable precautions.

Thibodeau shall not cause or allow visible emissions within 20 feet of ground level, measured as any level of opacity and not including water vapor, beyond the legal boundary of the property on which such emissions occur. Compliance with this standard shall be determined pursuant to 40 C.F.R. Part 60, Appendix A, Method 22.

[06-096 C.M.R. ch. 101, § 4(C)]

#### Equipment Relocation [06-096 C.M.R. ch. 115, BPT]

- Thibodeau shall provide written notification to the Bureau of Air Quality prior to relocation of any equipment carried on this license. It is preferred for notice of relocation to be submitted through the Department’s on-line e-notice at: www.maine.gov/dep/air/compliance/forms/relocation

Written notice may also be sent by mail. Notification sent by mail shall be sent to the address below:

Attn: Relocation Notice

Maine DEP

Bureau of Air Quality

17 State House Station

Augusta, ME 04333-0017

The notification shall include the license number in which the equipment is addressed, identification of the equipment moved, the address of the equipment’s new location, and the date the equipment will be moved.

- Written notification shall also be made to the municipality where the equipment will be relocated, except in the case of an unorganized territory where notification shall be made to the respective county commissioners. The notification to the Department shall include the date the municipality was notified.

#### Thibodeau shall keep a copy of this Order on site with the licensed equipment and ensure the operator(s) are familiar with the terms of this Order. [06-096 C.M.R. ch. 115, BPT]

#### If the Department determines that any parameter value pertaining to construction and operation of the emissions units, including but not limited to stack size, configuration, flow rate, emission rates, nearby structures, etc., deviates from what was submitted in the application or ambient air quality impact analysis for this air emission license, Thibodeau may be required to submit additional information. Upon written request from the Department, Thibodeau shall provide information necessary to demonstrate AAQS will not be exceeded, potentially including submission of an ambient air quality impact analysis or an application to amend this air emission license to resolve any deficiencies and ensure compliance with AAQS. Submission of this information is due within 60 days of the Department’s written request unless otherwise stated in the Department’s letter.

[06-096 C.M.R. ch. 115, § 2(O)]

DONE AND DATED IN AUGUSTA, MAINE THIS 20th DAY OF 	MARCH, 2026.

DEPARTMENT OF ENVIRONMENTAL PROTECTION

BY:							for

MELANIE LOYZIM, COMMISSIONER

The term of this license shall be ten (10) years from the signature date above.

[Note: If a renewal application, determined as complete by the Department, is submitted prior to expiration of this license, then pursuant to Title 5 M.R.S. § 10002, all terms and conditions of the license shall remain in effect until the Department takes final action on the license renewal application.]

PLEASE NOTE ATTACHED SHEET FOR GUIDANCE ON APPEAL PROCEDURES

Date of initial receipt of application:	8/27/2025

Date of application acceptance:	8/28/2025

This Order prepared by Jack Doran, Bureau of Air Quality.